# ==============================

# IMPORT LIBRARIES

# ==============================

import numpy as np

import pandas as pd

from scipy.signal import butter, filtfilt

# ==============================

# LOAD RAW DATA

# ==============================

emg_df = pd.read_excel(r"C:\Users\abish\Downloads\aakash main project\raw_EMG_data.xlsx")

imu_df = pd.read_excel(r"C:\Users\abish\Downloads\aakash main project\raw_IMU_data.xlsx")

# ==============================

# FILTER FUNCTIONS

# ==============================

def bandpass_filter(signal, fs, lowcut=20, highcut=450, order=4):

nyq = 0.5 * fs

low = lowcut / nyq

high = highcut / nyq

b, a = butter(order, [low, high], btype='band')

return filtfilt(b, a, signal)

def lowpass_filter(signal, fs, cutoff=6, order=4):

nyq = 0.5 * fs

normal_cutoff = cutoff / nyq

b, a = butter(order, normal_cutoff, btype='low')

return filtfilt(b, a, signal)

def rms_envelope(signal, window_size):

return np.sqrt(

np.convolve(signal**2,

np.ones(window_size)/window_size,

mode='same')

)

# ==============================

# EMG FILTERING

# ==============================

fs_emg = 1000 # Hz

emg_muscles = ["RF", "BF", "SEM", "VL", "GAS", "TA"]

emg_filtered = pd.DataFrame()

emg_filtered["Time_s"] = emg_df["Time_s"]

for muscle in emg_muscles:

raw_signal = emg_df[muscle].values

bp_signal = bandpass_filter(raw_signal, fs_emg)

rectified_signal = np.abs(bp_signal)

emg_filtered[muscle] = rectified_signal

# ==============================

# EMG RMS ENVELOPE (OPTIONAL)

# ==============================

window_ms = 50 # RMS window (ms)

window_samples = int(window_ms * fs_emg / 1000)

emg_rms = pd.DataFrame()

emg_rms["Time_s"] = emg_filtered["Time_s"]

for muscle in emg_muscles:

emg_rms[muscle] = rms_envelope(

emg_filtered[muscle].values,

window_samples

)

# ==============================

# IMU FILTERING

# ==============================

fs_imu = 100 # Hz

imu_joints = ["Hip_FlexExt_deg", "Knee_FlexExt_deg", "Ankle_DorsiPlantar_deg"]

imu_filtered = pd.DataFrame()

imu_filtered["Time_s"] = imu_df["Time_s"]

for joint in imu_joints:

imu_filtered[joint] = lowpass_filter(

imu_df[joint].values,

fs_imu

)

# ==============================

# SAVE FILTERED DATA

# ==============================

emg_filtered.to_excel(r"C:\Users\abish\Downloads\aakash main project\filtered data emg", index=False)

emg_rms.to_excel("EMG_RMS_envelope.xlsx"),

imu_filtered.to_excel(r"C:\Users\abish\Downloads\aakash main project\filtered data imu", index=False)

print("EMG and IMU filtration completed successfully.")


====================================================================================================================================================================

# ======================================

# EMG + IMU FILTRATION PIPELINE

# (Windows-safe, thesis-ready)

# ======================================

import numpy as np

import pandas as pd

import os

from scipy.signal import butter, filtfilt

# ======================================

# USER SETTINGS (ONLY CHANGE THIS)

# ======================================

BASE_PATH = r"C:\Users\abish\Downloads\aakash main project\raw data"

EMG_FILE = r"C:\Users\abish\Downloads\aakash main project\raw data\raw_EMG_data.xlsx"

IMU_FILE = r"C:\Users\abish\Downloads\aakash main project\raw data\raw_IMU_data.xlsx"

# ======================================

# LOAD DATA

# ======================================

emg_df = pd.read_excel(os.path.join(BASE_PATH, EMG_FILE))

imu_df = pd.read_excel(os.path.join(BASE_PATH, IMU_FILE))

# ======================================

# FILTER FUNCTIONS

# ======================================

def bandpass_filter(signal, fs, lowcut=20, highcut=450, order=4):

nyq = 0.5 * fs

low = lowcut / nyq

high = highcut / nyq

b, a = butter(order, [low, high], btype="band")

return filtfilt(b, a, signal)

def lowpass_filter(signal, fs, cutoff=6, order=4):

nyq = 0.5 * fs

normal_cutoff = cutoff / nyq

b, a = butter(order, normal_cutoff, btype="low")

return filtfilt(b, a, signal)

def rms_envelope(signal, window_size):

return np.sqrt(

np.convolve(

signal ** 2,

np.ones(window_size) / window_size,

mode="same"

)

)

# ======================================

# EMG FILTERING

# ======================================

fs_emg = 1000  # Hz

emg_muscles = ["RF", "BF", "SEM", "VL", "GAS", "TA"]

emg_filtered = pd.DataFrame()

emg_filtered["Time_s"] = emg_df["Time_s"]

for muscle in emg_muscles:

raw = emg_df[muscle].values

bp = bandpass_filter(raw, fs_emg)

emg_filtered[muscle] = np.abs(bp)

# ======================================

# EMG RMS ENVELOPE

# ======================================

window_ms = 50

window_samples = int(window_ms * fs_emg / 1000)

emg_rms = pd.DataFrame()

emg_rms["Time_s"] = emg_filtered["Time_s"]

for muscle in emg_muscles:

emg_rms[muscle] = rms_envelope(

emg_filtered[muscle].values,

window_samples

)

# ======================================

# IMU FILTERING

# ======================================

fs_imu = 100  # Hz

imu_joints = [

"Hip_FlexExt_deg",

"Knee_FlexExt_deg",

"Ankle_DorsiPlantar_deg"

]

imu_filtered = pd.DataFrame()

imu_filtered["Time_s"] = imu_df["Time_s"]

for joint in imu_joints:

imu_filtered[joint] = lowpass_filter(

imu_df[joint].values,

fs_imu

)

# ======================================

# SAVE OUTPUT FILES

# ======================================

os.makedirs(BASE_PATH, exist_ok=True)

emg_filtered.to_excel(os.path.join(BASE_PATH, r"C:\Users\abish\Downloads\aakash main project\Filtered data.xlsx"),

index=False

)

emg_rms.to_excel(

os.path.join(BASE_PATH, r"C:\Users\abish\Downloads\aakash main project\Filtered data\filtered data emg.xlsx"),

index=False

)

imu_filtered.to_excel(

os.path.join(BASE_PATH, r"C:\Users\abish\Downloads\aakash main project\Filtered data\filtered data imu.xlsx"),

index=False

)

print("✅ EMG & IMU filtration completed successfully.")

==================================================================================
**********************************************************************************
+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++