- EMG sampling rate = 1000 Hz

- IMU sampling rate = 100 Hz

- You already load raw data into NumPy arrays

🧩 STAGE 1 — FILTERING (RAW → CLEAN)

🟦 CELL 1: Import libraries

import numpy as np

from scipy.signal import butter, filtfilt, iirnotch, find_peaks

🟦 CELL 2: EMG filtering + smoothing

def bandpass_emg(signal, fs, lowcut=20, highcut=450, order=4):

nyq = 0.5 * fs

b, a = butter(order, [lowcut/nyq, highcut/nyq], btype='band')

return filtfilt(b, a, signal)

def notch_emg(signal, fs, freq=50, Q=30):

w0 = freq / (0.5 * fs)

b, a = iirnotch(w0, Q)

return filtfilt(b, a, signal)

def rms_envelope(signal, fs, window_ms=50):

window = int(window_ms * fs / 1000)

return np.sqrt(np.convolve(signal**2,

np.ones(window)/window,

mode='same'))

def preprocess_emg(raw_emg, fs):

emg = bandpass_emg(raw_emg, fs)

emg = notch_emg(emg, fs)

emg = np.abs(emg)              # rectification

emg = rms_envelope(emg, fs)    # smoothing

return emg

🟦 CELL 3: IMU filtering

def lowpass_imu(signal, fs, cutoff=10, order=4):

nyq = 0.5 * fs

b, a = butter(order, cutoff/nyq, btype='low')

return filtfilt(b, a, signal)

def preprocess_imu(raw_signal, fs):

return lowpass_imu(raw_signal, fs)

🧩 STAGE 2 — 6 MUSCLES + 8 PHASE SEGMENTATION (IMU ONLY)

🟦 CELL 4: Gait event detection (IMU gyro)

def detect_gait_events(gyro_signal, fs):

min_dist = int(0.5 * fs)

swing_peaks, _ = find_peaks(gyro_signal, distance=min_dist)

heel_strikes = []

for p in swing_peaks:

search = gyro_signal[p:p+int(0.3*fs)]

if len(search) > 0:

heel_strikes.append(p + np.argmin(search))

return np.array(heel_strikes)

🟦 CELL 5: Build gait cycles

def build_gait_cycles(heel_strikes):

return [(heel_strikes[i], heel_strikes[i+1])

for i in range(len(heel_strikes)-1)]

🟦 CELL 6: 8-phase segmentation

PHASE_RANGES = [

(0.00, 0.02), (0.02, 0.12), (0.12, 0.30), (0.30, 0.50),

(0.50, 0.60), (0.60, 0.73), (0.73, 0.87), (0.87, 1.00)

]

def segment_8_phases(gait_cycles):

all_phases = []

for start, end in gait_cycles:

length = end - start

phases = [(start + int(p1*length),

start + int(p2*length))

for p1, p2 in PHASE_RANGES]

all_phases.append(phases)

return all_phases

🟦 CELL 7: Select 6 muscles

MUSCLES = ['RF', 'VL', 'BF', 'TA', 'GAS', 'SOL']

def select_muscles(emg_dict, side):

return {m: emg_dict[side][m] for m in MUSCLES}

🧩 STAGE 3 — FEATURE EXTRACTION (PHASE-WISE)

🟦 CELL 8: EMG feature extraction

def emg_features(signal):

return {

'RMS': np.sqrt(np.mean(signal**2)),

'MAV': np.mean(np.abs(signal)),

'iEMG': np.sum(np.abs(signal))

}

🟦 CELL 9: Phase-wise EMG features

def phasewise_emg_features(emg_signal, phases):

features = []

for start, end in phases:

seg = emg_signal[start:end]

features.append(emg_features(seg))

return features

🟦 CELL 10: IMU feature extraction

def imu_features(signal):

return {

'Range': np.max(signal) - np.min(signal),

'RMS': np.sqrt(np.mean(signal**2))

}

🟦 CELL 11: Phase-wise IMU features

def phasewise_imu_features(imu_signal, phases):

features = []

for start, end in phases:

seg = imu_signal[start:end]

features.append(imu_features(seg))

return features

🧠 FINAL MENTAL MODEL (VERY IMPORTANT)

RAW DATA

↓

STAGE 1: FILTERING

↓

STAGE 2: 6 MUSCLES + 8 PHASES (IMU)

↓

STAGE 3: FEATURE EXTRACTION

↓

(Next → symmetry → BCI)

STAGE 1: FILTERING

✔ EMG → band-pass + smoothing

✔ IMU → 4th-order Butterworth LPF

STAGE 2: 6 MUSCLES + 8 PHASES

✔ IMU-based gait event detection

STAGE 3: FEATURE EXTRACTION

✔ Phase-wise EMG & IMU features

PART 1 — LEFT–RIGHT SYMMETRY COMPUTATION

🟦 CELL 1: Symmetry Index function

This is the standard symmetry index used in gait studies.

def symmetry_index(left, right, eps=1e-8):

"""

Returns symmetry score between 0 and 1

1 = perfect symmetry

"""

return 1 - np.abs(left - right) / (left + right + eps)

🟦 CELL 2: EMG symmetry (muscle-wise, phase-wise)

Expected input structure

# emg_features[side][muscle][phase]['RMS']

def emg_phase_symmetry(emg_features, muscles, feature_name='RMS'):

phase_symmetry = []

for phase in range(8):

muscle_scores = []

for muscle in muscles:

L = emg_features['Left'][muscle][phase][feature_name]

R = emg_features['Right'][muscle][phase][feature_name]

muscle_scores.append(symmetry_index(L, R))

phase_symmetry.append(np.mean(muscle_scores))

return np.array(phase_symmetry)

Output

S_EMG = [S1, S2, ..., S8]

🟦 CELL 3: IMU symmetry (phase-wise)

Expected input structure

# imu_features[side][phase]['Range']

def imu_phase_symmetry(imu_features, feature_name='Range'):

phase_symmetry = []

for phase in range(8):

L = imu_features['Left'][phase][feature_name]

R = imu_features['Right'][phase][feature_name]

phase_symmetry.append(symmetry_index(L, R))

return np.array(phase_symmetry)

Output

S_IMU = [S1, S2, ..., S8]

🧩 PART 2 — BILATERAL COORDINATION INDEX (BCI)

🟦 CELL 4: Phase-wise BCI computation

This is where EMG + IMU are fused.

def compute_bci_phasewise(S_EMG, S_IMU, w_emg=0.5, w_imu=0.5):

return w_emg * S_EMG + w_imu * S_IMU

Output

BCI_phase = [BCI1, BCI2, ..., BCI8]

🟦 CELL 5: Overall BCI score

Single number for whole gait cycle.

def compute_bci_overall(BCI_phase):

return np.mean(BCI_phase)

🧪 FULL USAGE EXAMPLE (IMPORTANT)

muscles = ['RF', 'VL', 'BF', 'TA', 'GAS', 'SOL']

S_EMG = emg_phase_symmetry(emg_features, muscles)

S_IMU = imu_phase_symmetry(imu_features)

BCI_phase = compute_bci_phasewise(S_EMG, S_IMU)

BCI_overall = compute_bci_overall(BCI_phase)

print("Phase-wise BCI:", BCI_phase)

print("Overall BCI:", BCI_overall)